Location:________________________________Name__________________________Date__________

Basic First Responder Training for Incidents Involving Grain Storage, 
Processing, and Handling Facilities

Pre-Test

- What is the average number of grain entrapments that occur in the U.S. per year?

- 5-10

- 15-25

- 30-40

- 50-60

- Hazards potentially faced by first responders to a grain entrapment rescue include:

  - Falls

  - Exposure to toxic dust

  - Over exertion/heat stress

  - All of the above

- The type of grain most frequently involved in flowing grain entrapment is:

- Soybeans

- Corn

- Wheat

- Oats

- On-farm grain storage structures are generally considered:

- Exempt from federal OSHA regulations

- Permit-required confined spaces

- Toxic environments

- All of the above

- A grain entrapment victim may experience:

- Dust inhalation

- Lower extremity trauma from auger contact

- Hypothermia

- All of the above

- The most frequent cause of death associated with grain engulfment is:

- Hypothermia

- Crushing injuries

- Asphyxiation

- Impact injuries

- Is it possible for a person completely engulfed beneath the surface of a grain pile to survive for more than fifteen minutes?

- Yes

- No

- Pulling a victim entrapped chest deep in grain can cause severe injuries.

- True

- False

- The most common type of grain entrapment occurs when a worker falls through a crusted surface of grain into an empty cavity below that surface.

- True

- False

- The angle of repose for grain can be 25-30% depending on moisture content.

  - True

  - False

- When cutting grain bin panels, which of the following personal protective equipment is needed?

  - Eye protection

  - Hearing protection

  - Respiratory protection

  - All of the above

- The first step that should be taken at the scene of a grain entrapment is to:

  - Enter the bin to access the situation

  - Begin cutting open the structure

  - Lockout or disconnect all power sources

  - Turn on unloading system

- The grain near the walls of a bin moves faster during unloading, thus creating a greater risk of entrapment near the walls than in the center.

- True

- False

- A typical on-farm grain bin is designed with the following potential anchor points:

- Inside bin ladder

- Roof top hatch ring

- Stirring auger or grain distributor supports

- None of the above

- The purpose of a grain rescue tube is to:

  - Reduce pressure on the victim

  - Isolate the victim from the surrounding grain

  - Protect the victim from avalanching grain

  - All of the above

- Cutting open a 100,000 bushel capacity grain storage structure can lead to:

- Catastrophic failure of structure

- Movement of victim within the grain

- Engulfment of first responders

- All of the above

- A torch should never be used to cut holes in a grain bin because of the potential for a grain fire or dust explosion.

- True

- False

- A good way to rescue a partially entrapped victim is to attach a harness to him/her and start the unloading auger to remove grain below him/her.

- True

- False

- The most likely source of entanglement inside a grain bin is the:

  - Stirring auger

  - Sweep auger

  - In-floor auger

  - Portable auger

- A grain vacuum machine removes grain at such a slow rate that it poses no risk of entrapment to a first responder.

- True

- False