### ME 323: Mechanics of Materials 							Quiz 5

## Name:_____________________

Q1 (10 Points): A beam of a T-shape cross section is subject to a linearly distributed load shown below. b=2 in., h=6 in. Determine the moment in the middle section of the beam (i.e., 6 ft from A). Also, within this cross section, find:

- The location of the neutral axis of the cross section.

- The second area moment of the cross section.

- The largest magnitude of stress in the cross section

(i)

y=(A1y1+A2y2)A1+A2=2*63+6*272*2*6=5 in  from the bottom of the cross-section

(ii)

I=I10+I20

I=2*6312+6*25-32+6*2312+2*67-52

I=136 in4

(iii)  First find reactions:

ΣFy=Ay+By-720=0

ΣMA=12By-8*720=0

By=480 lb

Ay=240 lb

Find reactions at cross section at 6 ft:

ΣM=M6-6Ay+2*180=0

→M6=1080 lb*ft=12960 lb*in

Find max flexural stress:

Max distance from origin will be at the bottom (y=5)

σmaxx=6 ft=M6yI=12960*5136=426.5 psi