RADIOACTIVITY

SpecTech ST-150 Nuclear Scaler

Indiana State Standards:

C.1.24	Recognize and describe nuclear changes.

C.1.41	Describe the role of light, heat, and electrical energies in physical, chemical, and nuclear changes.

INTRODUCTION

The nuclei of certain atoms are stable and under ordinary circumstances, stable nuclei do not undergo change.  The nuclei of other atoms are unstable.  These nuclei undergo change spontaneously, that is, without outside help.  When unstable nuclei undergo change they give off radiation.  Atoms which have unstable nuclei are radioactive and are called radioisotopes or radionuclides.

The change that an unstable nucleus undergoes is called disintegration or decay.  When unstable nuclei disintegrate, or decay, certain particles - alpha or beta particles - or bundles of energy called gamma radiation, are emitted.

Atoms and molecules in the path of radiation are ionized, that is, they are stripped of electrons.  That means alpha particles, beta particles, gamma radiation and other nuclear emissions have enough energy to remove some electrons from atoms or molecules with which they collide.  Positively charged particles and free electrons are left behind after the collisions.

Devices that are used to detect radioactivity are based on the ionizing ability of radiation.  One such device is the Geiger-Muller tube which is connected to a counter and is commonly referred to as a nuclear scaler.  When a charged particle or gamma radiation enters the Geiger-Muller tube, it ionizes many of the argon gas atoms in the tube.  The electrons are attracted to the anode and the argon ions are attracted to the cathode.  This produces a surge of current which can be counted by the scaler.

PURPOSE

To investigate the nature of radioactivity and the effect of time, distance, and shielding materials on various radioactive sources using a G.M. tube with counter (generally referred to as a scaler.)

MATERIALS

Geiger-Muller tube (nuclear scaler)

Radioactive sources for alpha particles, beta particles, and gamma radiation

Forceps

Various shielding materials as provided by the instructor

SAFETY

Use forceps or tweezers when handling all radioactive material.

Follow the standard safety procedures as explained by your teacher.

PROCEDURE

NOTE:  WHEN NOT IN USE, ALL RADIOACTIVE SOURCES AND OTHER RADIOACTIVE MATERIAL, SUCH AS LUMINOUS WATCH DIALS, SHOULD BE PLACED AT LEAST 1 METER AWAY FOR THE SCALER WHILE MEASUREMENTS ARE BEING TAKEN.

PART A:  BACKGROUND RADIATION

1. Turn the scaler on by pressing button on the back of the instrument.  The instrument does not need to

warm up.

2. Press the H.V. button.  Set the voltage on the nuclear scaler at 440 volts by pressing the up/down

buttons.

3. Place the empty shield tray in the first shelf slot from the top.

4. Press the time button.  Set the time counter interval for 30 seconds by pressing the up/down buttons.

The red light will be on.

5. Press the count button.  When the time stops hit the time button again. The display will show the count.

The scaler will count for the set time interval and stop automatically at the end of the time.  Pressing

the time button again will give you the count.

6. Take 2 more counts for the background reading.

7. Average the three counts for the background radiation and record your average.

8. Answer Questions A-1 and A-2 on your data sheet.

PART B.  OBSERVING RANDOMNESS OF DISINTEGRATION

1. Set the time counter interval for 30 sec.

2. Using forceps, place the alpha source on the plastic holder and place it on the first shelf position from

the top.

3. Press the <<COUNT>> button and record the count following the procedure used in part A.

4. Repeat the count of the alpha source on the first shelf two more times.  Record your results.

5. Answer Questions B-1 and B-2 on your data sheet.

PART C.  THE EFFECT OF TIME ON THE AMOUNT OF EXPOSURE TO RADIATION

1. Set the time counter interval for 30 sec.

2. Using forceps, place the beta source in the plastic holder and place it in the third shelf position from

the top.

3. Take the count.

4. Set the time counter interval for 60 sec.

5. Take the count.

6. Set the time counter interval for 100 sec.

7. Take the count.

8. Answer Questions C-1, C-2, and C-3 on your data sheet.

PART D.  THE EFFECT OF DISTANCE ON THE AMOUNT OF EXPOSURE TO RADIATION

1. Set the time counter interval to 30 sec.

2. Using forceps, place the beta source in the plastic holder and place it in the second shelf position from

the top.

3. Press the <<COUNT>> button.  When the time stops, press the time button again.  Record the number

of counts on the display.

4. Move the plastic tray containing the beta source to the third shelf position from the top.

5. Press the <<COUNT>> button.  When the time stops, press the time button again.  Record the number

of counts on the display.

6. Move the plastic tray containing the beta source to the fourth position, then the fifth position and

finally the sixth position, counting the radiation at each level.

7. Answer Question D-1 on your data sheet.

PART E.  THE EFFECT OF SHIELDING ON THE AMOUNT OF EXPOSURE TO RADIATION.

SECTION 1.  BETA PARTICLE RADIATION

1. Set the time counter interval to 30 sec.

2. Using forceps, place the beta source in the plastic holder and place it in the second shelf position from

the top.

3. Press the <<COUNT>> button.  When the time stops, press the time button again.  Record the number

of counts on the display.

4. Place a piece of notebook paper in the first shelf position from the top.

5. Take the count.

6. Replace the notebook paper with another shielding material.

7. Use other shielding materials and take the count as directed by your teacher.

SECTION 2.  ALPHA PARTICLE RADIATION

1. Follow the procedure outlined in Part E, Section 1 but use an alpha source instead of a beta source.

SECTION 3.  GAMMA RADIATION

- Follow the procedure outlined in Part E, Section 1 but use a gamma source instead of the beta source.

- Answer Questions E-1 through E-4 on your data sheet.

NAME ____________________			Kit Number __________

CLASS PERIOD __________			Instrument Number __________

RADIOACTIVITY DATA SHEET

PART A:  BACKGROUND RADIATION

| Trial Number | Count/ 30 s |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| Average Background |  |

Question A-1:  What is being counted?

Question A-2:  Where did it come from?

PART B:  RANDOMNESS OF DISINTEGRATION

| Trial Number | Count/ 30 s |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |

Question B-1. Compare the three values.  Are they exactly the same, very close, or very different?

Question B-2. What do these results indicate about the nature of radioactive decay?

PART C:  THE EFFECT OF TIME ON THE AMOUNT OF EXPOSURE TO RADIATION

| Time Interval | Count |
| --- | --- |
| 30 seconds |  |
| 60 seconds |  |
| 100 seconds |  |

Question C-1. From your data, what conclusions can you make concerning time and the amount of

exposure to radiation received?

Question C-2. Predict the approximate number of counts you would measure in 3 minutes.

Question C-3. Approximately how many counts would you observe in 24 hours?

PART D.  THE EFFECT OF DISTANCE ON THE AMOUNT OF EXPOSURE TO RADIATION

| Shelf Position | Distance From Source | Count |
| --- | --- | --- |
| 2nd From Top | 2 cm |  |
| 3rd From Top | 3 cm |  |
| 4th From Top | 4 cm |  |
| 5th From Top | 5 cm |  |
| Bottom | 6 cm |  |

Question D-1. From your data, why is it preferable to build a nuclear power plant in a sparsely

populated area rather than close to a big city?

PART E.  THE EFFECT OF SHIELDING ON THE AMOUNT OF EXPOSURE TO RADIATION

| Shielding Material | Beta Count | Alpha Count | Gamma Count |
| --- | --- | --- | --- |
| Count from 2nd shelf- no shielding |  |  |  |
| Notebook paper |  |  |  |

Question E-1.Which source (beta, alpha, or gamma) is most easily shielded?

Question E-2.Which source is the least easily shielded?

Question E-3.Which of the shielding materials was the most effective?

Question E-4.Which of the shielding materials was the least effective?

DATA ANALYSIS AND CALCULATIONS

1. Using your textbook as a resource, describe an alpha particle, a beta particle, and gamma radiation.

2. Is there a correlation between the mass of the particle radiated and the ease with which it is shielded?

3. Predict the effect on the count for each source if the source had been shielded by two pieces of

notebook paper rather than just one.

4. Predict the effect on the count for each source if the source had been shielded by five pieces of

notebook paper rather than just one.

5. Graph of distance vs count.

i. Using your data for PART D, subtract the average background count from the count at each level.  This adjusted count is that which is produced by the source.

ii. Plot a graph of the distance from the source vs. the adjusted count.  The distance should go on the horizontal axis and the adjusted count should go on the vertical axis.

6. Describe the relationship between the amount of radiation and the distance from the source.

7. The activity of a radioactive source is 500 counts at a distance of 10 cm.  What would be the

approximate activity if the source was moved to a distance of 20 cm?

8. How would your exposure to radiation from the sun differ if you were on the planet Mercury or on

the planet Pluto?

TEACHERS' GUIDE

RADIOACTIVITY

CLASSROOM USAGE

This is considered a discovery lab.  It may be appropriate for physical science as well as practical and college prep chemistry classes.

CURRICULUM INTEGRATION

Nuclear Energy - this may be done anytime while studying the unit.

PREPARATION

Shielding materials have to be provided.  Some things which can be used are plastic wrap, aluminum foil, a piece of fabric. lead foil, window glass, wood (tongue depressors), tissue, and cotton.

GETTING READY

It is important to familiarize yourself with the set-up and operation of the nuclear scaler.  Our scalers work at the 440 volt setting, but it would be best to verify that this is a good operating voltage.  You will probably want the students to work in pairs.

TIME

This experiment will probably require two lab periods.

SAFETY AND DISPOSAL

The radioactive sources will not provide any unusual hazard for the students.  They should handle the samples with forceps.  If there is a concern about having touched a sample, washing the hands will be sufficient.

VARIATIONS

This lab may be expanded, cut back, or rearranged to fit your schedule.  You may want the students to bring in some different materials.  Some students may want to change the time intervals.