Ohm’s Law Problems

Use the following equations to help you solve the problems below:

I = V/R     V =  I*R     R = V/I

- What is the current when 3 V is applied across a 6Ω resistor?

2.   What is the voltage when a 1,200 Ω resistor passes a current of 0.2 A.

3.    What is the voltage of a circuit with a resistance of 250 Ω and a current of 0.95 A?

4.    What is the voltage of a line with a resistance of 10 Ω and a current of 20 A?

5.     What is the current in a line with 200 V and a resistance of 40 Ω?

6.     What is the resistance in a line with 400 volts and a current of 100 A?

7.     What is the voltage in a line with a resistance of 10 Ω and a current of 40 A?

8.     What is the current in a line with 200 volts and a resistance of 80 Ω?

9.     What is the resistance in a line with 400 volts and a current of 200 A?

10.     What is the resistance in a line with a current of 50 A and 400 V?

Ohm’s Law Problems - Key

- What is the current when 3 V is applied across a 6 Ω resistor?

I = V/R = 3 V / 6 Ω = 0.5 A

2.   What is the voltage when a 1,200 Ω resistor passes a current of 0.2 A.

V = I * R = 0.2 A * 1,200 Ω = 240 V

3.    What is the voltage of a circuit with a resistance of 250 Ω and a current of 0.95 A?

V = I * R = 0.95 A * 250 Ω = 237.5 V

4.    What is the voltage of a line with a resistance of 10 Ω and a current of 20 A?

V = I * R = 20 A * 10 Ω = 200 V

5.     What is the current in a line with 200 V and a resistance of 40 Ω?

I = V/R = 200 V / 40 Ω = 5 A

6.     What is the resistance in a line with 400 volts and a current of 100 A?

R = V/I = 400 V / 100 A = 4 Ω

7.     What is the voltage in a line with a resistance of 10 Ω and a current of 40 A?

V = I * R = 40 A * 10 Ω = 400 V

8.     What is the current in a line with 200 volts and a resistance of 80 Ω?

I = V/R = 200 V / 80 Ω = 2.5 A

9.     What is the resistance in a line with 400 volts and a current of 200 A?

R = V/I = 400 V / 200 A = 2 Ω

10.     What is the resistance in a line with a current of 50 A and 400 V?

R = V/I = 400 V / 50 A = 8 Ω